16x16 HDMI Matrix IP Address Setting Steps

* Before Setting the IP, users need to well get the 16x16 HDMI matrix well connected with the control PC by the LAN cable

- Configuration for the LAN Port:

- Open the ZNetCom2 software, showing as below:

Click Search, then will find the 16x16 HDMI matrix below:

After found out the equipment, need to: Click “ZNE-300T” first and then click “Get Info”, then the configuration for the matrix will be showing on the left side of the control interface.

If need to change the settings, will need to enter the password 88888 first.

After changing the parameter, will need to click “Apply Change”

Below are the default parameters:

- Work Type：TCP Server

- Work Port：15000

- Baudrate：115200

- Data Bits：8

- Stop Bits：1

- TCP Connection：4.

- IP：192.168.0.178