Operation of the DIP switch for Input card:

When pull to “ON/ VE” area as: 0, and pull to number area as: 1

| D1, D2 | Input source selection | D1 D2 0 0 CVBS input 0 1 YPbPr input 1 0 VGA input 1 1 DVI Input |
| --- | --- | --- |
| D3, D4, D5 | Internal Resolution Selection | D3 D4 D5 0 0 0 1024*768 0 0 1 1360*768 0 1 0 1920*1200 0 1 1 720P/60 1 0 0 Null 1 0 1 Null 1 1 0 1080P/ 50 1 1 1 1080P/ 60 |
| D6 | Audio Selection | 0: Force to 3.5mm audio input 1: Auto adapt |
| D7 | Auto recognize | 0: Turn on 1: Turn off |
| D8 | IR Switch | 0: Turn off 1: Turn on |

Example: If need to use the internal resolution 1080P/60Hz,

only need to pull D3, D4, D5 as 111

If need to force to 3.5mm audio input,

only need to pull D6 as 0

Operation of the DIP switch for Output card:

When pull to “ON/ VE” area as: 0, and pull to number area as: 1

| D1 | Color Space Selection | 0: RBG 1: YUV |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D2 D3 D4 D5 D6 | Output Resolution Selection | D2 | D3 | D4 | D5 | D6 |  |
|  |  | 0 | 0 | 0 | 0 | 0 | 1024*768/60 |
|  |  | 0 | 0 | 0 | 0 | 1 | 800*600/60 |
|  |  | 0 | 0 | 0 | 1 | 0 | 1280*800/60 |
|  |  | 0 | 0 | 0 | 1 | 1 | 1280*1024/60 |
|  |  | 0 | 0 | 1 | 0 | 0 | 1360*768/60 |
|  |  | 0 | 0 | 1 | 0 | 1 | 1366*768/60 |
|  |  | 0 | 0 | 1 | 1 | 0 | 1440*1050/60 |
|  |  | 0 | 0 | 1 | 1 | 1 | 1440*900/60 |
|  |  | 0 | 1 | 0 | 0 | 0 | 1680*1050/60 |
|  |  | 0 | 1 | 0 | 0 | 1 | 1600*1200/60 |
|  |  | 0 | 1 | 1 | 0 | 0 | 1680*1050/60 |
|  |  | 0 | 1 | 1 | 0 | 1 | 1400*900/75 |
|  |  | 0 | 1 | 1 | 1 | 0 | 640*480/75 |
|  |  | 0 | 1 | 1 | 1 | 1 | 800*600/75 |
|  |  | 1 | 0 | 0 | 0 | 0 | 480i/60 |
|  |  | 1 | 0 | 0 | 0 | 1 | 576i/50 |
|  |  | 1 | 0 | 0 | 1 | 0 | 480P/60 |
|  |  | 1 | 0 | 0 | 1 | 1 | 576P/50 |
|  |  | 1 | 0 | 1 | 0 | 0 | 1280*720/24 |
|  |  | 1 | 0 | 1 | 0 | 1 | 1280*720/25 |
|  |  | 1 | 0 | 1 | 1 | 0 | 1280*720/30 |
|  |  | 1 | 0 | 1 | 1 | 1 | 1280*720/50 |
|  |  | 0 | 0 | 0 | 1 | 1 | 1280*720/60 |
|  |  | 1 | 1 | 0 | 0 | 1 | 1080i/50 |
|  |  | 1 | 1 | 0 | 1 | 0 | 1080i/60 |
|  |  | 1 | 1 | 0 | 1 | 1 | 1080P/24 |
|  |  | 1 | 1 | 1 | 0 | 0 | 1080P/25 |
|  |  | 1 | 1 | 1 | 0 | 1 | 1080P/30 |
|  |  | 1 | 1 | 1 | 1 | 0 | 1080P/50 |
|  |  | 1 | 1 | 1 | 1 | 1 | 1080P/560 |
| D7 | Image Mirror function | 0: Rotate off 180 degrees 1: Turn off mirror function |  |  |  |  |  |
| D8 | IR switch | 0: Turn on 1: Turn off |  |  |  |  |  |

Example:

If need to use RGB color space output, only need to pull D1 as 0

If need to use 1920*1200/60Hz, only need to pull D2, D3, D4, D5, D6 as 01010

If need need to rotate off the image 180°, only need to pull D7 as 0