## RS232 Control Commands For the New 8x8 HDMI Matrix

RS232 cable with straight-through connection(USB-RS232 can be used directly to control)

Communication protocol:

Baud rate: 9600

Data bit: 8

Stop bit: 1

Check bit: None

| Commands | Explanation | Function description |
| --- | --- | --- |
| YXZ. | Y=1,2,3,4...... Z=1,2,3,4...... | Switch Input Y to Output Z Eg. “1X2.” means switch Input 1 to output 2 |
| SaveY. | Y=1,2,3,4...... | Save current status to scene Y Eg. “Save2.” means saving current status to Scene 2 |
| RecallY. | Y=1,2,3,4...... | Recall the saved scene Y Eg. “Recall2.” means recall the saved Scene 2 |
| Buzzeron. | Beep sound | Buzzer on |
| Buzzeroff. |  | Buzzer off |

Note:

- Every command ends with a period “.” and it can’t be missing.

- The letter can be capital or small letter.